Task 3.1

- Aim of the study using Fuzzy Logic.

This study aims to develop an intelligent HVAC control system utilizing fuzzy logic to automatically change the air conditioner temperature in response to environmental conditions and number of people in the room. The objective is to enhance thermal comfort, decrease energy consumption, and lower the manual intervention in temperature regulation decisions (Dewantoro et al., 2020).

- Advantages of employing a fuzzy system for HVAC control.

Fuzzy systems in HVAC have clear advantages when it comes to energy use. Fuzzy systems also help deal with vagueness, and uncertainty in real-world data by replicating how people think to make flexible decisions.

- Reduces energy consumption: Changes the temperature automatically based on how many people there are and the environmental condition. According to Dewantoro et al. (2020), raising the setpoint by 1 degree saves about 6.14 percent of energy.

- Eliminates manual control No longer relies on users to set the temperature. This makes things more consistent.

- Handles vague input: People have different ways of feeling temperature. Fuzzy logic utilizes linguistic variables, including cold, warm, and hot, to model uncertainty (Dewantoro et al., 2020).

- Improves Comfort: Keeps the temperature inside at a comfortable level based on the current conditions instead of fixed settings.

- Low computational cost: Compared to complex AI models, fuzzy systems run efficiently on microcontrollers like Arduino.

- Explanation of fuzzy system pipeline

- Inputs:

  - Outdoor temperature

  - Number of occupants

- Output:

  - The air conditioner temperature setpoint, which is then passed on to the AC unit as an infrared command.

- Membership functions, fuzzification, degree of membership

  - Membership functions are used to transform each input into fuzzy sets.

Temperature: Cold, Warm, Hot

Occupants: Few, Medium, Many

- Fuzzification converts each crisp input into degrees of membership in these sets, for example assigning partial membership to Warm and Hot when the outdoor temperature lies between 27.5 °C and 35 °C, or to Medium and Many when the number of occupants lies between 25 and 40 (Dewantoro et al. 2020)

- Fuzzy Rule: The system uses IF THEN rules based on human reasoning.

If outdoor temperature is Cold and occupants are Few, then setpoint is High.

If outdoor temperature is Hot and occupants are Many, then setpoint is Low.

Intermediate combinations, such as Warm with Medium occupants, yield Middle setpoints.

- Defuzzification Stage: The system takes the fuzzy rules and turns them into crisp values. It does this by using a weighted average method to find the final temperature setting for the air conditioner. Then the system sends this temperature setting to the air conditioner as a signal. The air conditioner uses this signal to adjust its setpoint.

- Hardware sensors used in the experiment setup.

- The DHT22 temperature sensor is to measure outdoor temperature

- The HC-SR04 sensors to count how many people are entering and leaving the room.

- The Arduino Mega2560 - to process the fuzzy logic inference.

- The infrared transmitter sends signals to the air conditioner.

- Operating cycles of the air conditioner using the fuzzy system

The air conditioner operates in cycles based on fuzzy logic decisions. The compressor switches  between ON at 100 percent and OFF at 0 percent due to fixed frequency operation. Initially, when room temperature is far from the setpoint, the compressor runs continuously. As it gets closer to the desired temperature it turns ON for a time and turns OFF more often. This helps save energy.

Tests show that over time the ON periods get shorter as the room gets closer, to the temperature. The system keeps the room comfortable while using the compressor less. The air conditioner controls the temperature in a way, which uses less energy and keeps the room steady at a comfortable temperature. This helps lower energy consumption. Keeps the indoor conditions stable.

Reference

Dewantoro, G., Nugraha, B.E. and Setiaji, F.D. (2020) ‘A fuzzy logic‑based automation toward intelligent air conditioning systems’, Kinetik: Game Technology, Information System, Computer Network, Computing, Electronics, and Control, 5(4), pp. 335–344. doi:10.22219/kinetik.v5i4.1135